Contents

Tank Design and Construction - New API 650 Tanks - Part 195 Requirements	2

Tank Design and Construction - New API 650 Tanks - Foundation Design	6

Tank Design and Construction - New API 650 Tanks - Floor Design	8

Tank Design and Construction - New API 650 Tanks - Shell Design	10

Tank Design and Construction - New API 650 Tanks - Welding & NDT	11

Tank Design and Construction - New API 650 Tanks - Appurtenances & Nozzles	15

Tank Design and Construction - New API 650 Tanks - Hydrostatic Testing	16

Tank Design and Construction - New Tank Piping - Construction	17

NTSB Recommendations Review	19

## HL Inspections - HL Inspections

## Tank Design and Construction - New API 650 Tanks - Part 195 Requirements

1. New Aboveground Breakout Tank Specifications Does the process for new aboveground atmospheric breakout tanks require tank design and construction to meet the requirements of 195.132(b)(3)? (TDC.650REGS.TANKSPEC.P). 195.132(b)(3) (API Std 650 (13th Edition)). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

2. New Aboveground Breakout Tank Specifications Do the design records and drawings indicate new aboveground atmospheric breakout tanks are designed and constructed to the specifications required by 195.132(b)(3)? (TDC.650REGS.TANKSPEC.R). 195.132(b)(3) (API Std 650 (13th Edition)). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

9. Breakout Tank CP - System Design (API RP 651) Does the process for new aboveground breakout tanks require cathodic protection system design to conform with API RP 651 (4th Edition), Sections 6.2 and 6.3, as required by 195.565? (TDC.650REGS.CPDESIGN.P). 195.565 (195.563(d); 195.132(b)(3); API RP 651 (4th Edition), Section 6.3.4; API RP 651 (4th Edition), Section 6.3.5; API RP 651 (4th Edition), Section 7.2.1; API RP 651 (4th Edition), Section 11.4). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

10. Breakout Tank CP - System Design (API RP 651) Do records demonstrate new aboveground breakout tanks have cathodic protection installed as required by 195.565? (TDC.650REGS.CPDESIGN.R). 195.565 (195.404(c); 195.563(d); 195.589(a); 195.589(b); 195.589(c); API RP 651 (4th Edition), Section 6.3.4; API RP 651 (4th Edition), Section 6.3.5; API RP 651 (4th Edition), Section 7.2.1; API RP 651 (4th Edition), Section 11.4). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

11. Breakout Tank CP - System Design (API RP 651) Do field observations confirm new breakout tanks have cathodic protection installed in accordance with 195.565? (TDC.650REGS.CPDESIGN.O). 195.565 (195.563(d); API RP 651 (4th Edition), Section 6.3.4; API RP 651 (4th Edition), Section 6.3.5; API RP 651 (4th Edition), Section 7.2.1). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

12. Breakout Tank - Venting or Pressure/Vacuum Relief Does the process for new aboveground breakout tanks require normal / emergency (pressure/vacuum) relief venting to be provided for each tank in accordance with 195.264(d) and (e)? (TDC.650REGS.RELIEFVENT.P). 195.264(e) (195.264(d); API Std 650 (13th Edition); API Std 2000 (7th Edition)). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

13. Breakout Tank - Venting or Pressure/Vacuum Relief Do design records indicate normal / emergency (pressure/vacuum) relief venting was provided for each for new aboveground breakout tank in accordance with 195.264(d) and (e)? (TDC.650REGS.RELIEFVENT.R). 195.264(e) (195.264(d); API Std 650 (13th Edition); API Std 2000 (7th Edition)). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

14. Breakout Tank - Venting or Pressure/Vacuum Relief Do field observations confirm normal / emergency (pressure/vacuum) relief venting was provided for each for new aboveground breakout tank in accordance with 195.264(d) and (e)? (TDC.650REGS.RELIEFVENT.O). 195.264(e) (195.264(d); API Std 650 (13th Edition); API Std 2000 (7th Edition)). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

15. Breakout Tank Overfill Protection Does the new tank design require product level alarm devices to be installed to indicate a rise of the liquid in the tank to a level above the normal and overfill protection levels in accordance with 195.428(c)? (TDC.650REGS.OVERFILLPROT.P). 195.428(c) (195.402(c); 195.132(b)(3); API Recommended Practice 2350 (5th Edition), Section 4.6; API Recommended Practice 2350 (5th Edition), Section 4.8). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

16. Breakout Tank Overfill Protection Do records indicate product level alarm devices were installed and set to alarm at a level above the normal and overfill protection levels in accordance with 195.428(c)? (TDC.650REGS.OVERFILLPROT.R). 195.428(c) (195.404(a); 195.404(b); 195.404(c); 195.132(b)(3); API Recommended Practice 2350 (5th Edition), Section 4.6; API Recommended Practice 2350 (5th Edition), Section 4.8). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

17. Breakout Tank Overfill Protection Do field observations confirm product level alarm devices were installed and set to alarm at the design levels (level above the normal and overfill protection levels) in accordance with 195.428(c)? (TDC.650REGS.OVERFILLPROT.O). 195.428(c) (195.132(b)(3); API Recommended Practice 2350 (5th Edition), Section 4.6; API Recommended Practice 2350 (5th Edition), Section 4.8). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

18. Breakout Tank Overfill Protection - Testing & Acceptance Does the design process require testing and inspection of the overfill protection system upon initial installation? (TDC.650REGS.OVERFILLTESTING.P). 195.428(c) (API Recommended Practice 2350 (5th Edition), Section 4.8.1; API Recommended Practice 2350 (5th Edition), Section 4.8.2(a); API Recommended Practice 2350 (5th Edition), Section 4.8.7; API Recommended Practice 2350 (5th Edition), Appendix H.5.3). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

19. Breakout Tank Overfill Protection - Testing & Acceptance Do records indicate testing and inspection of the overfill protection system was performed upon initial installation? (TDC.650REGS.OVERFILLTESTING.R). 195.428(c) (API Recommended Practice 2350 (5th Edition), Section 4.8.2(a); API Recommended Practice 2350 (5th Edition), Section 4.8.7; API Recommended Practice 2350 (5th Edition), Section 4.8.1; API Std 650 (13th Edition), Appendix H.5.3). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

20. Breakout Tank Overfill Protection - Testing & Acceptance Do field observations indicate testing and inspection of the tank overfill protection system was performed? (TDC.650REGS.OVERFILLTESTING.O). 195.428(c) (API Recommended Practice 2350 (5th Edition), Section 4.8.2(a); API Std 650 (13th Edition), Appendix H.5.3). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

21. Breakout Tank Overfill Protection - SCADA Does the process require initial testing of applicable SCADA overfill protection systems for each new tank? (TDC.650REGS.OVERFILLSCADA.P). 195.446(c)(2) (195.428(d); API Recommended Practice 2350 (5th Edition)). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

22. Breakout Tank Overfill Protection - SCADA Do records indicate initial testing of applicable SCADA overfill protection systems for each new tank was conducted? (TDC.650REGS.OVERFILLSCADA.R). 195.446(c)(2) (195.428(d); API Recommended Practice 2350 (5th Edition)). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

23. Breakout Tank Overfill Protection - SCADA Do field observations confirm initial testing was conducted for applicable SCADA overfill protection systems for each new tank? (TDC.650REGS.OVERFILLSCADA.O). 195.446(c)(2) (195.428(d); API Recommended Practice 2350 (5th Edition)). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

29. Breakout Tanks - Impoundment Does the process for new aboveground breakout tanks require impoundment(s) to meet the impoundment requirements of 195.264 in the event of tank spillage or failure? (TDC.650REGS.IMPOUNDMENT.P). 195.264(a) (195.264(b); 195.264(c); 195.264(d); 195.264(e); NFPA 30-2012). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

30. Breakout Tanks - Impoundment Do records indicate that new aboveground breakout tanks include impoundment(s) meet the requirements of 195.264 in the event of tank spillage or failure? (TDC.650REGS.IMPOUNDMENT.R). 195.264(a) (195.264(b); 195.264(c); 195.264(d); 195.264(e); NFPA 30-2012). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

31. Breakout Tanks - Impoundment Do field observations confirm that impoundment(s) for new aboveground breakout tanks were installed in accordance with the requirements of 195.264? (TDC.650REGS.IMPOUNDMENT.O). 195.264(a) (195.264(b); 195.264(c); 195.264(d); 195.264(e); NFPA 30-2012). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

## Tank Design and Construction - New API 650 Tanks - Foundation Design

1. Seismic Tank Design (API 650 Appendix E) For tanks located in regions that may be subject to seismic ground motion (earthquakes), does the process require adherence to API Std 650 (13th Edition), Appendix E - "Seismic Design of Storage Tanks" and a site-specific seismic study (Appendix E.4.2.1)? (TDC.650FDN.SEISMICDESIGN.P). 195.132(b)(3) (API Std 650 (13th Edition), Appendix E.1; API Std 650 (13th Edition), Appendix E.3; API Std 650 (13th Edition), Appendix E.4; API Std 650 (13th Edition), Appendix E.5; API Std 650 (13th Edition), Appendix E.6; API Std 650 (13th Edition), Appendix E.7; ASCE 7). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

2. Seismic Tank Design (API 650 Appendix E) For tanks located in regions that may be subject to SEISMIC ground motion (earthquakes), do records (design package) indicate a site-specific seismic study was performed (Appendix E.4.2.1) and the seismic requirements of API Std 650 (13th Edition), Appendix E, are incorporated? (TDC.650FDN.SEISMICDESIGN.R). 195.132(b)(3) (API Std 650 (13th Edition), Appendix E.1; API Std 650 (13th Edition), Appendix E.3; API Std 650 (13th Edition), Appendix E.4; API Std 650 (13th Edition), Appendix E.5; API Std 650 (13th Edition), Appendix E.6; API Std 650 (13th Edition), Appendix E.7; ASCE 7). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

3. Seismic Tank Design (API 650 Appendix E) For tanks located in regions that may be subject to SEISMIC ground motion (earthquakes), do field observations indicate that the seismic design requirements from API Std 650 (13th Edition), Appendix E, were implemented and/or installed? (TDC.650FDN.SEISMICDESIGN.O). 195.132(b)(3) (API Std 650 (13th Edition), Appendix E.1; API Std 650 (13th Edition), Appendix E.3; API Std 650 (13th Edition), Appendix E.4; API Std 650 (13th Edition), Appendix E.5; API Std 650 (13th Edition), Appendix E.6; API Std 650 (13th Edition), Appendix E.7; ASCE 7). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

4. Foundation - General Design, Subsurface Conditions, and Ringwall Are the tank specifications complete for the proper procedure/aspect of tank foundation and ringwall design and construction? (TDC.650FDN.FDNDESIGN.P). 195.132(b)(3) (API Std 650 (13th Edition), Appendix B.1; API Std 650 (13th Edition), Appendix B.2; API Std 650 (13th Edition), Appendix B.3; API Std 650 (13th Edition), Appendix B.4; API Std 650 (13th Edition), Appendix E.7.6; API Std 650 (13th Edition), Section 5.3.1.2; API Std 650 (13th Edition), Section 5.11.4). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

5. Foundation - General Design, Subsurface Conditions, and Ringwall Do field observations confirm the tank foundation and ringwall were constructed and/or installed in accordance with the design specifications? (TDC.650FDN.FDNDESIGN.O). 195.132(b)(3) (API Std 650 (13th Edition), Appendix B.2; API Std 650 (13th Edition), Appendix B.3; API Std 650 (13th Edition), Appendix B.4). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

6. Foundation Design - Subsurface Conditions Do records demonstrate all of the subsurface conditions and factors that affect foundation design were investigated? (TDC.650FDN.SUBSURFACE.R). 195.132(b)(3) (API Std 650 (13th Edition), Appendix B.2.2; API Std 650 (13th Edition), Appendix B.2.3; API Std 650 (13th Edition), Appendix B.2.4). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

7. Foundation Design - Subsurface Conditions Do field observations confirm the subsurface conditions and factors match the foundation design? (TDC.650FDN.SUBSURFACE.O). 195.132(b)(3) (API Std 650 (13th Edition), Appendix B.2.2; API Std 650 (13th Edition), Appendix B.2.3; API Std 650 (13th Edition), Appendix B.2.4). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

## Tank Design and Construction - New API 650 Tanks - Floor Design

1. Floor - Lap-Welded Bottom Plate Joints Does the tank floor design specify that lap-welded bottom plates and joint welding conform to API Std 650 (13th Edition), Section 5.1.5.4? (TDC.650FLOOR.BOTTOMJOINTS.P). 195.132(b)(3) (API Std 650 (13th Edition), Section 5.1.5.4). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

2. Floor - Lap-Welded Bottom Plate Joints Do records indicate that lap-welded bottom plates and joint welding conforms to the design specifications? (TDC.650FLOOR.BOTTOMJOINTS.R). 195.132(b)(3) (API Std 650 (13th Edition), Section 5.1.5.4). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

3. Floor - Lap-Welded Bottom Plate Joints Do field observations confirm that lap-welded bottom plates and joint welding conforms to the design specifications? (TDC.650FLOOR.BOTTOMJOINTS.O). 195.132(b)(3) (API 650, Section 5.1.5.4). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

4. Floor - Weld Pass Restrictions Does the tank floor design specify weld pass restrictions as defined in API Std 650 (13th Edition), Section 5.1.3.6? (TDC.650FLOOR.WELDPASSRES.P). 195.132(b)(3) (API Std 650 (13th Edition), Section 5.1.3.6). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

5. Floor - Weld Pass Restrictions Do field observations confirm weld pass restrictions were in accordance with API Std 650 (13th Edition), Section 5.1.3.6? (TDC.650FLOOR.WELDPASSRES.O). 195.132(b)(3) (API Std 650 (13th Edition), Section 5.1.3.6). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

9. Floor - Shell-to-Bottom Fillet Welds Does the tank floor design specify that shell-to-bottom plates welding conform to API Std 650 (13th Edition), Section 5.1.5.7? (TDC.650FLOOR.SHELLBOTTOMWELDS.P). 195.132(b)(3) (API Std 650 (13th Edition), Section 5.1.5.7). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

10. Floor - Shell-to-Bottom Fillet Welds Do field observations confirm shell-to-bottom plates welding conforms to the design specifications? (TDC.650FLOOR.SHELLBOTTOMWELDS.O). 195.132(b)(3) (API Std 650 (13th Edition), Section 5.1.5.7). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

## Tank Design and Construction - New API 650 Tanks - Shell Design

4. Shell - Plate Design Stress Does the operator's design procedures require determination of maximum allowable product design stress and maximum allowable hydrostatic test stress based on permissible plate materials in API Std 650 (13th Edition), Tables 5-2a and 5-2b? (TDC.650SHELL.PLATESTRESS.P). 195.132(b)(3) (API Std 650 (13th Edition), Section 5.6.2.1; API Std 650 (13th Edition), Section 5.6.2.2). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

5. Shell - Plate Design Stress Do records (design package) indicate the maximum allowable product design stress and maximum allowable hydrostatic test stress meet the requirements of the design specifications? (TDC.650SHELL.PLATESTRESS.R). 195.132(b)(3) (API Std 650 (13th Edition), Section 5.6.2.1; API Std 650 (13th Edition), Section 5.6.2.2). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

6. Shell - Course Thickness Method Do records (design package) indicate the selected course thickness method met the design requirements? (TDC.650SHELL.THICKNESSMETHOD.R). 195.132(b)(3) (API Std 650 (13th Edition), Section 5.6.3; API Std 650 (13th Edition), Section 5.6.4; API Std 650 (13th Edition), Section 5.6.5). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

## Tank Design and Construction - New API 650 Tanks - Welding & NDT

1. Welding - Welding Procedure Specifications Do the tank welding specifications require the erection/fabrication manufacturer to prepare welding procedure specifications that comply with ASME BPVC code section IX-2007 (and any additional provisions of API Std 650 (13th Edition))? (TDC.650WELDING.PROCEDURES.P). 195.214 (195.132(b)(3); API Std 650 (13th Edition), Section 9.2.1; API Std 650 (13th Edition), Section 7.2.1.10; API Std 650 (13th Edition), Section 5.1.5.2; ASM BPVC Section IX-2007). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

2. Welding - Welding Procedure Specifications Do records indicate the tank erection/fabrication manufacturer prepared welding procedure specifications that comply with ASME code section IX-2007(and any additional provisions of API Std 650 (13th Edition))? (TDC.650WELDING.PROCEDURES.R). 195.214(b) (195.132(b)(3); API Std 650 (13th Edition), Section 9.2.1; API Std 650 (13th Edition), Section 7.2.1.10; API Std 650 (13th Edition), Section 5.1.5.2; API Std 650 (13th Edition), Appendix W.1.3; ASME BPVC Section IX-2007). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

3. Welding - Welding Procedure Specifications Do field observations indicate the tank erection/fabrication manufacturer followed the welding procedure specifications (WPS)? (TDC.650WELDING.PROCEDURES.O). 195.214(a) (195.214(b); 195.132(b)(3); API Std 650 (13th Edition), Section 9.2.1; API Std 650 (13th Edition), Section 7.2.1.10; API Std 650 (13th Edition), Section 5.1.5.2). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

4. Welding - Interpretation of Weld Inspections Do the operator's procedures require the proper interpretation of each weld inspection, under 195.234(c), to ensure the acceptability of each weld under 195.228? (TDC.650WELDING.INTERPRETATION.P). 195.234(c) (195.228; 195.132(b)(3)). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

5. Welding - Qualification of Welders Do the tank welding specifications require each welder to be qualified for welding in accordance with ASME BPVC Section IX-2007 and the welder qualification requirements of API Std 650 (13th Edition), Section 9.3? (TDC.650WELDING.WELDERQUAL.P). 195.132(b)(3) (195.222; API Std 650 (13th Edition), Section 9.3). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

6. Welding - Qualification of Welders Do records indicate each welder was qualified for welding in accordance with ASME BPVC Section IX-2007 and the welder qualification requirements of API Std 650 (13th Edition), Section 9.3? (TDC.650WELDING.WELDERQUAL.R). 195.132(b)(3) (195.222; API Std 650 (13th Edition), Section 9.3). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

7. Welding - Qualification of Welders Is each welder observed in the field properly qualified for welding in accordance with ASME BPVC Section IX-2007 and the welder qualification requirements of API Std 650 (13th Edition), Section 9.3? (TDC.650WELDING.WELDERQUAL.O). 195.234(c) (195.222; 195.132(b)(3)). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

8. Welding - Welding Weather Conditions Do field observations confirm operator took appropriate measures to accommodate welding during adverse weather condition and/or cold temperatures, and specifically prohibited welding on wetted surfaces and during high winds? (TDC.650WELDING.WELDINGWEATHER.O). 195.224 (195.132(b)(3); API Std 650 (13th Edition), Section 7.2.1.3). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

9. Welding - Radiographic Inspection of Shell Butt Welds Do the operator's procedures require radiographic inspection of shell butt-welds and insertions plates (i.e., tombstones) to conform with API Std 650 (13th Edition), Sections 8.1.2 and 5.7.8.11? (TDC.650WELDING.RADIOGRAPHIC.P). 195.234(b) (195.132(b)(3); API Std 650 (13th Edition), Section 8.1.1; API Std 650 (13th Edition), Section 8.1.2; API Std 650 (13th Edition), Section 5.7.8.11; API Std 650 (13th Edition), Section 9.4). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

10. Welding - Radiographic Inspection of Shell Butt Welds Do records indicate radiographic inspection was conducted on required shell butt-welds, annular-plate butt-welds, and flush-type connections with butt-welds? (TDC.650WELDING.RADIOGRAPHIC.R). 195.234(b) (195.132(b)(3); API Std 650 (13th Edition), Section 8.1.1; API Std 650 (13th Edition), Section 8.1.2; API Std 650 (13th Edition), Section 5.7.8.11; API Std 650 (13th Edition), Section 9.4). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

11. Welding - Non-Destructive Testing Personnel Certification Do records indicate all Non-Destructive Testing (NDT) personnel are qualified and certified by the manufacturer as meeting the required certification and/or API Std 650 (13th Edition) requirements? (TDC.650WELDING.NDTEXAMINER.R). 195.234(b) (195.132(b)(3); API Std 650 (13th Edition), Section 8.1.3.2; API Std 650 (13th Edition), Section 8.2.3; API Std 650 (13th Edition), Section 8.3.2.4; API Std 650 (13th Edition), Section 8.4.3; API Std 650 (13th Edition), Section 8.5.1; API Std 650 (13th Edition), Section 8.6). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

12. Welding - Non-Destructive Testing Personnel Certification Do field observations indicate all Non-Destructive Testing (NDT) personnel are qualified and certified by the manufacturer as meeting the required certification and/or API Std 650 (13th Edition) requirements? (TDC.650WELDING.NDTEXAMINER.O). 195.234(b) (195.132(b)(3); API Std 650 (13th Edition), Section 8.1.3.2; API Std 650 (13th Edition), Section 8.2.3; API Std 650 (13th Edition), Section 8.3.2.4; API Std 650 (13th Edition), Section 8.4.3; API Std 650 (13th Edition), Section 8.5.1; API Std 650 (13th Edition), Section 8.6). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

13. Welding - Repair of Defective Welds Do the welding specifications provide criteria for weld acceptability and weld defects that must be removed and repaired? (TDC.650WELDING.ACCEPTREPAIR.P). 195.132(b)(3) (195.230; API Std 650 (13th Edition), Section 7.4; API Std 650 (13th Edition), Section 8.1.7; API Std 650 (13th Edition), Section 8.5.2; API Std 650 (13th Edition), Section 8.5.3). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

14. Welding - Repair of Defective Welds Do records indicate the criteria for weld acceptability and weld defects that must be removed and repaired were followed? (TDC.650WELDING.ACCEPTREPAIR.R). 195.132(b)(3) (195.230; API Std 650 (13th Edition), Section 7.4; API Std 650 (13th Edition), Section 8.1.7; API Std 650 (13th Edition), Section 8.5.2; API Std 650 (13th Edition), Section 8.5.3). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

15. Welding - Repair of Defective Welds Do field observations of welding NDE match the criteria for weld acceptability and weld defects that must be removed and repaired? (TDC.650WELDING.ACCEPTREPAIR.O). 195.132(b)(3) (195.230; API Std 650 (13th Edition), Section 7.4; API Std 650 (13th Edition), Section 8.1.7; API Std 650 (13th Edition), Section 8.5.2; API Std 650 (13th Edition), Section 8.5.3). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

17. Welding - Spot Radiographic Inspection Do records indicate tank shell spot radiographic inspection on a per tank basis was conducted in accordance with the radiographic specifications? (TDC.650WELDING.SPOTRADIOGRAPHS.R). 195.132(b)(3) (API Std 650 (13th Edition), Section 8.1.2.2; API Std 650 (13th Edition), Section 8.1.2.3). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

19. Welding - Shell Vertical Joints Alignment Do field observations confirm shell vertical joints conform with API 650, Section 5.1.5.2(a) and (b) and Figure 5-1? (TDC.650WELDING.VERTICALJOINTS.O). 195.132(b)(3) (API 650, Section 5.1.5.2). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

20. Welding - Shell-to-Bottom Welds Examination Do field observations confirm the initial weld pass on the shell-to-bottom weld was examined for its entire circumference, both visually and using one of the approved methods in API Std 650 (13th Edition), Section 7.2.4.1? (TDC.650WELDING.SHELL2BOTTOMEXAM.O). 195.132(b)(3) (API Std 650 (13th Edition), Section 7.2.4). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

21. Welding - Shell Reinforcing Plate Do field observations confirm reinforcing plates were being tested to 15 psig pneumatic pressure between the tank shell and the reinforcing plate? (TDC.650WELDING.SHELLREINFORCEPLATE.O). 195.132(b)(3) (API Std 650 (13th Edition), Section 7.3.5). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

## Tank Design and Construction - New API 650 Tanks - Appurtenances & Nozzles

2. Appurtenances - Shell Openings Do field observations confirm shell openings, manholes, and reinforcements were installed in accordance with the design specifications? (TDC.650APPURT.SHELLOPENINGS.O). 195.132(b)(3) (API Std 650 (13th Edition), Section 5.7.1; API Std 650 (13th Edition), Section 5.7.2; API Std 650 (13th Edition), Section 5.7.5.1). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

## Tank Design and Construction - New API 650 Tanks - Hydrostatic Testing

1. Hydrotesting - New Tank Shell Hydrotesting Does the process for new aboveground breakout tanks require hydrostatic leak testing of tanks in accordance with 195.307(c)? (TDC.650HYDRO.HYDROTEST.P). 195.307(c) (195.310(a); 195.310(b); 195.132(b)(3); API Std 650 (13th Edition), Section 7.3.6; API Std 650 (13th Edition), Section 7.3.7; API Std 650 (13th Edition), Appendix L.3, Line 14). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

2. Hydrotesting - New Tank Shell Hydrotesting Do testing records indicate the new atmospheric aboveground breakout tank(s) hydrostatic leak testing was successfully conducted in accordance with 195.307(c)? (TDC.650HYDRO.HYDROTEST.R). 195.307(c) (195.310(a); 195.310(b); 195.132(b)(3); API Std 650 (13th Edition), Section 7.3.6; API Std 650 (13th Edition), Section 7.3.7; API Std 650 (13th Edition), Appendix L.3, Line 14; API Std 650 (13th Edition), Appendix W.1.5). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

3. Hydrotesting - New Tank Shell Hydrotesting Do field observations confirm atmospheric breakout tank hydrostatic leak testing was successfully conducted in accordance with 195.307 and the testing specifications? (TDC.650HYDRO.HYDROTEST.O). 195.307(c) (195.310(a); 195.310(b); 195.132(b)(3); API Std 650 (13th Edition), Section 7.3.6; API Std 650 (13th Edition), Section 7.3.7; API Std 650 (13th Edition), Appendix L.3, Line 14). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

4. Hydrotesting - Floating Roof (EFR/IFR) Floatation Test Do field observations confirm the floating roof (internal or external) and its accessories operated without damage to the floating roof, the seal, and any tank appurtenances? (TDC.650HYDRO.FLOATINGROOF.O). 195.132(b)(3) (API Std 650 (13th Edition), Appendix C.4.3; API Std 650 (13th Edition), Appendix H.4.1). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

6. Hydrotesting - Manufacturer Certification of Tank Construction IAW API 650 Do records indicate the tank manufacturer certified the completed tank was successfully constructed in accordance with API Std 650 (13th Edition) and attached a nameplate to the tank shell? (TDC.650HYDRO.TANKCERTIFIC.R). 195.132(b)(3) (API Std 650 (13th Edition), Section 10.1; API Std 650 (13th Edition), Section 10.3). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

7. Hydrotesting - Manufacturer Certification of Tank Construction IAW API 650 Do observations confirm the certification nameplate was attached to the tank shell? (TDC.650HYDRO.TANKCERTIFIC.O). 195.132(b)(3) (API Std 650 (13th Edition), Section 10.1). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

## Tank Design and Construction - New Tank Piping - Construction

6. Breakout Tank Piping - Pressure Testing Where tank piping and/or manifolds are installed in association with new breakout tank construction, does the process require pressure testing of all piping, fittings, and components in accordance with 195.302, 195.304, and 195.305? (TDC.TKPIPING.TANKPIPINGTEST.P). 195.302(a) (195.304; 195.305(a); 195.306(a); 195.306(b); 195.306(c); 195.306(d); 195.305(b)). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

7. Breakout Tank Piping - Pressure Testing Where tank piping and/or manifolds are installed in association with new breakout tank construction, do records indicate all piping, fittings, and components were pressure tested in accordance with 195.302, 195.304, and 195.305? (TDC.TKPIPING.TANKPIPINGTEST.R). 195.302(a) (195.304; 195.305(a); 195.306(a); 195.306(b); 195.306(c); 195.306(d); 195.305(b)). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

8. Breakout Tank Piping - Pressure Testing Do field observations of tank piping and/or manifolds pressure testing confirm that all piping, fittings, and components were pressure tested in accordance with 195.302, 195.304, and 195.305? (TDC.TKPIPING.TANKPIPINGTEST.O). 195.302(a) (195.304; 195.305(a); 195.306(a); 195.306(b); 195.306(c); 195.306(d); 195.305(b)). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

11. Breakout Tanks - Installation & Testing of Piping Protective Devices Prior to Service Does the process require the installation and initial testing of tank piping pressure limiting devices, relief valves, pressure regulators, or other items of pressure control prior to place the aboveground breakout tank into service? (TDC.TKPIPING.PROTDEVICETEST.P). 195.428(a) (195.402(c)(3)). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

12. Breakout Tanks - Installation & Testing of Piping Protective Devices Prior to Service Do records indicate tank piping pressure limiting devices, relief valves, pressure regulators, or other items of pressure control were installed and tested prior to placing the aboveground breakout tank into service? (TDC.TKPIPING.PROTDEVICETEST.R). 195.428(a) (195.404(c)(3)). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

13. Breakout Tanks - Installation & Testing of Piping Protective Devices Prior to Service Do field observations confirm tank piping pressure limiting devices, relief valves, pressure regulators, or other items of pressure control were installed and tested prior to placing the aboveground breakout tank into service? (TDC.TKPIPING.PROTDEVICETEST.O). 195.428(a) (195.402(c)(3)). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

## NTSB Recommendations Review

1. NTSB Recommendations Review Does the operator have procedures in place for reviewing NTSB Recommendations? (GENERIC.NTSB.NTSBREVIEW.P). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

2. NTSB Recommendations Review Do the records verify operator conducted reviews of NTSB recommendations and implemented appropriate actions? (GENERIC.NTSB.NTSBREVIEW.R). Sat+; Sat; Concern; Unsat; NA; NC. Notes.

Except as required to be disclosed by law, any inspection documentation, including completed protocol forms, summary reports, executive summary reports, and enforcement documentation are for internal use only by federal or state pipeline safety regulators. Some inspection documentation may contain information which the operator considers to be confidential. In addition, supplemental inspection guidance and related documents in the file library are also for internal use only by federal or state pipeline safety regulators (with the exception of documents published in the federal register, such as advisory bulletins). Do not distribute or otherwise disclose such material outside of the state or federal pipeline regulatory organizations. Requests for such information from other government organizations (including, but not limited to, NTSB, GAO, IG, or Congressional Staff) should be referred to PHMSA Headquarters Management.