Summary of

Electric to Natural Gas Water Heater Conversion Program

| Assumptions |  |  |
| --- | --- | --- |
| Description | Value | Source |
| Inflation Rate | 3.0% | Staff Analysis |
| Cost of Capital/ Discount Rate | 9.0% | Staff Analysis |
| Measure Life | 13 Years | http://www.aceee.org/Consumerguide/waterheating.htm |
| Electric Avoided Cost | See Attached | Staff Analysis (attached) |
| Natural Gas Avoided Cost | See Attached | Staff Analysis (attached) |
| Measure Cost | $900 | http://www.aceee.org/Consumerguide/waterheating.htm |
| Avoided Cost Alternate Fuel Eq. | $8004 | http://www.aceee.org/Consumerguide/waterheating.htm |
| Electricity Usage Change | -4,875 kWh | http://www.aceee.org/Consumerguide/waterheating.htm |
| Natural Gas Usage Change | +25 Mcf5 | http://www.aceee.org/Consumerguide/waterheating.htm |
| Incentive | $900 | Company |
| Net-to-Gross Ratio | 1.0 | TRM Order, page 17 |
| Participation | 1 | Company |
| Results |  |  |
| Test | Net Benefit | Benefit/Cost Ratio |
| Total Resource Cost Test | $6,149 | 2.80 |
| Participant Test | $7,049 | 3.07 |
| Rate Impact Measure Test | $1,610 | 1.17 |
| Program Administrator Cost Test | $5,3499 | 2.57 |
| Total Energy Savings | 32,333,287 Btus | - |

From the staff avoided cost spreadsheet; the escalation rate assumed for natural gas distribution costs.

I conservatively assume a 100% load factor for water heating (Staff’s calculation assumes 53%), but I think this is reasonable for this particular evaluation and obviates the need to make a separate estimate of demand savings from the measure.

See my escalation in out-years to accommodate a 13-year measure life.

The ACEEE value is for November, 2007.  I escalated at 3% for two years to develop 2009 value.

Consistent with California Energy Commission assumptions and the assumed efficiency factors (.67/.97) of the TRM.

Assumed to be equal to the cost of the measure ($900) to minimize free-ridership.

Evaluated 1 participant, on the assumption that we are focused on the TRC ratio and that ratio will remain unchanged whether we have 1 or 1,000,000 participants.

Test result details attached as spreadsheet “WH Conversion Evaluation.”

Assumes that the avoided cost is equal to the retail rate.

See calculations attached in the spreadsheets “source-to-site” and “Energy Savings.”